#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <string.h>

#include <getopt.h>

#include <sys/types.h>

#include <sys/socket.h>

#include <arpa/inet.h>

/* defines */

#define DEFBUFSIZE (4096)

#define DEFPORT    (5001)

#define MAXBUFSIZE (32*1024)

#define UDP_MODE 0

#define TCP_MODE 1

#define PF_WIZNET PF_BLUETOOTH

char buffer[MAXBUFSIZE] = {0,};

static void usage(void)

{

fprintf(stderr, "\n");

fprintf(stderr, "usage: loopback [option]\n\n");

fprintf(stderr, "Options:\n");

fprintf(stderr, "-h               loopback help\n");

fprintf(stderr, "-u               UDP loopback mode\n");

fprintf(stderr, "-t               TCP loopback mode\n");

fprintf(stderr, "-w               PF_WIZNET loopback mode(Default 'PF_INET')\n");

fprintf(stderr, "-b <size>        buffer size(Default 4096)\n");

fprintf(stderr, "-p <port>        port number(Default 5001)\n");

exit(1);

}

int main (int argc, char *argv[])

{

int bufsize=DEFBUFSIZE;

struct sockaddr_in sock_in;

struct sockaddr addr;

int family=0;

int sd, fd;

int addr_len;

int opt;

int ret;

int proto_mode = -1;

int proto_type;

short port = DEFPORT;

bzero((char *)&sock_in, sizeof (sock_in));

bzero((char *)&addr, sizeof (addr));

/* command line argument option */

while( (opt=getopt(argc, argv, "huwtb:p:")) != EOF )

{

switch(opt)

{

case 'h':

usage();

return 0;

break;

case 't':

if(proto_mode >= 0)

{

fprintf(stderr, "Invalid Option.\n");

return -1;

}

proto_mode = TCP_MODE;

break;

case 'u':

if(proto_mode >= 0)

{

fprintf(stderr, "Invalid Option.\n");

return -1;

}

proto_mode = UDP_MODE;

break;

case 'b':

bufsize = abs(atoi(optarg));

break;

case 'p':

port = abs(atoi(optarg));

break;

case 'w':

family = PF_WIZNET;

break;

}

}

if(proto_mode < 0)

{

fprintf(stderr, "Invalid Option. Input protocal mode option.(-t or -u)\n");

return -1;

}

if(proto_mode)

proto_type = SOCK_STREAM;

else

proto_type = SOCK_DGRAM;

if(!family)

family = PF_INET;

/* create socket */

sd = socket(family, proto_type, 0);

if (sd < 0) {

perror("socket");

return -1;

}

sock_in.sin_family = family;

sock_in.sin_addr.s_addr = htonl(INADDR_ANY);

sock_in.sin_port = htons(port);

/* bind server port to socket */

if (bind(sd, (struct sockaddr*)&sock_in, sizeof (sock_in)) < 0) {

perror("bind");

return -1;

}

if(proto_mode)

{

/* listen */

if(listen(sd, 0) < 0)

{

perror("listen");

return -1;

}

/* accept */

addr_len = sizeof(addr);

fd = accept(sd, &addr, &addr_len);

if (fd < 0) {

perror("cannot accept connection ");

close(sd);

return -1;

}

}

printf("loopback start!\n");

if(proto_mode)

{

while(1)

{

/* recv data from client */

ret = recv(fd, buffer, bufsize, 0);

if(ret < 0)

{

perror("recv");

return -1;

}

/* send data(=recv data) to client */

ret = send(fd, buffer, ret, 0);

if(ret < 0)

{

perror("send");

return -1;

}

}

} else {

while(1)

{

addr_len = sizeof(addr);

ret = recvfrom(sd, buffer, bufsize, 0, &addr, &addr_len);

if(ret < 0)

{

perror("recvfrom");

return -1;

}

ret = sendto(sd, buffer, ret, 0, &addr, addr_len);

if(ret < 0)

{

perror("sendto");

return -1;

}

}

}

close(sd);

return 0;

}