How to switch betwen USB driver for WindowsXP stack and USB driver for BlueSuite applications

WindowsXP to Bluesuite

- Disconnect module

- Run usbpurger.exe (available in the zip or in the BlueSuite’s installation directory, normally

C:\Program Files\CSR\BlueSuite)

- (In rare cases, you might have to manually edit the Windows’ registry file at this point to remove all instances

where Vid_0a12&Pid_0001 appears, as permission settings may prevent usbpurger.exe to remove entries)

- Move the bth.inf (and delete the bth.pnf files), if still existing, from C:\WINDOWS\inf into another

folder or just rename it to something like bth.inf.temp

- Connect the module and install driver as follows

Instead of using the drivers in the zip (like in the window above), use preferably the drivers directly from BlueSuite’s installation directory, as seen in the next image:

At this point the device is installed:

And properly working:

Bluesuite to WindowsXP

- Disconnect the module

- Run usbpurger.exe (available in the zip or in the BlueSuite’s installation directory, normally

C:\Program Files\CSR\BlueSuite)

- (In rare cases, you might have to manually edit the Windows’ registry file at this point to remove all instances

where Vid_0a12&Pid_0001 appears, as permission settings may prevent usbpurger.exe to remove entries)

- Be sure to have recovered the bth.inf in the C:\WINDOWS\inf (you may as well use the one in the zip)

- Connect the module and install driver as follows.

At this point the device is installed (“Generic Bluetooth Radio” may appear if the standard bth.inf was used):

And properly working: